22

Team

Design Exchange 2021

This report submission is approved by:

Nzali Chitindi

Emmanuel Fashae

Ma-sum Abdul-Hamid

Fatima Naji Ali Abdullah Almansoori

Aishah Ahmed Mohammed Abdullateif

Contents

Executive Summary	3

Problem Statement and Definition	3

Solution Description	4

Technical Overview	6

Key Stakeholders	7

Expected Outcomes	8

Cost Summary	8

Calculations - Cost of Production	9

Program Timeline	10

References	10

## Executive Summary

## Problem Statement and Definition

Malawi currently has a very high electricity demand to supply ratio despite the country’s consideration of diverse energy production options. The estimated per capita electricity consumption is marked at 93kWh which is significantly lower than the 500-kWh global critical threshold. This major deficit in power supply greatly has slowed down the overall growth of the country's business sector and has led to considerable losses in commercial revenue. According to the data from the World Bank, Malawi loses up to 7 percent of its national GDP annually to electricity rationing commonly known as load shedding. The current energy mix in Malawi is dominated by thermal energy (generated from burning of charcoal and wood fuels). The patronage of this energy production sector has led to mass deforestation, desertification, forest cover depletion, and an increase in emissions of greenhouse gases and other pollutants into the environment. As such, the thermal energy-centric nature of the energy mix contributes greatly to the acceleration of the ever-changing global climate.

Malawi has been unable to rescue its energy crisis despite its consideration of alternative energy sources in hydroelectricity, solar energy, wind energy and so on.  Particularly, the generation of hydroelectric power, for which the country is heavily reliant on the Shire River and the Lake Malawi, is woefully inadequate in its quest to reduce the considerable power deficit. According to the Environmental Protection Agency (EPA), electricity generation constitutes about 25 percent of greenhouse gas emissions.  The effects of climate change on Malawi’s economy, although secondary, have been catastrophic. According to a study by the international monetary fund, the country has the fourth highest percentage of people living in extreme poverty in the world. What part does climate change play in this?

The rampancy of primary ramifications of climate change such as increasing temperatures both on land and in water, ocean acidification and rapidly changing precipitation patterns have led to huge losses through food and crop damage and the grave endangerment of aquatic life. Overall, the consequently low food and crop yield have negatively impacted the country’s agricultural economy.  The impacts of these circumstances are dire because, according to a study by the world bank, agriculture occupies 86% of Malawi’s workforce, makes up 38% of its GDP and 90% of its export earnings. With the aforementioned effects on the daily livelihoods of Malawians (since fishing and farming are the primary sources of income for most of the citizens) and the greater economy (through depreciation in government revenue and foreign exchange earnings), Climate change has indirectly contributed substantially to the poverty levels in Malawi.

## Solution Description

As Team Freedom Urban, we aim to develop a system that seeks to remedy the current energy crisis in peri-urban areas in Malawi with high entrepreneurial potential. Our solution will seek to reverse the adverse effects of the rapidly changing climate in Malawi and the greater Sub-Saharan Africa through a highly adjustable electricity generation system that greatly reduces the national overreliance on biomass energy and hydroelectricity.

Our mission targets SMEs with 50 or less employees. These businesses include saloons, barbershops, welding shops, electrical shops, mechanical, T.V repairing shops, restaurants, butcheries, among others. The number of local businesses under this program initially, depending on the targeted quantity of electricity to be supplied per business, will range from 350 to 500. From the projected energy scenario of Malawi, the projected electricity demand will be 874 MW by 2020 and 1193 MW by 2025 [7]. With a population of 19.39 million consuming a forecasted 874 MW of electricity, we deduced an electricity demand per capita of 45W. Going by these numbers, we conclude that SMEs under our program will typically consume about 2.25KW annually.

Carbon footprints:

We focus our analysis of the business carbon footprints through three different scopes:

• Scope 1: All forms of combustion by the business itself. For most of Malawi’s local businesses, this comprises coal burning for cooking, fuel burning in generators and mechanical devices.

• Scope 2: This is a wider scope that encapsulates the footprints from Scope 1 and additional footprints from purchased electricity (generated from the coal mines).

• Scope 3: This scope comprises carbon footprints from the business supply chain. This includes carbon emissions through transportation, from the use of any products being traded, and from the businesses’ investments.

As of 2016, with a population of 17.2 million, Malawi had a carbon footprint of 0.11 tons per capita [8].  Through deduction by proportions, we propose the carbon footprint of Malawi is 0.12 tons per capita.  Through this evaluation, we can then infer that the total carbon footprint of an SME in scope 1 will be at least 6 tons annually (0.12*50).

In a pie-chart illustration of the percentages of CO2 emissions in Malawi by sector, the power industry was seen to constitute 11.2% of the 1,815,598

tons in 2016 (203,347). Biomass energy (burning of charcoal and wood fuel) provides 94% of Malawi’s energy [9]. Going solely by these numbers, the total carbon footprint from biomass energy could be approximated as 191,146 tons (0.94*203,347). Through deduction by proportions, we can deduce the carbon footprint of an SME in Malawi as approximately 0.01 tons ((191146*45)/874000000). The total carbon footprint for each SME annually in scope 2 will therefore be 0.13 tons.

There is no definite way of estimating the carbon footprints from Scope 3. As such, the total carbon footprint for SMEs that will be considered in our solution will be 0.13 tons.

Our solution involves the distribution of solar-generated power through solar photovoltaic panels, the promotion of local SMEs, and the assistance of aspiring entrepreneurs in Lilongwe through the provision of controlled internet connectivity and city-wide advertisement.

1. Electricity generation with tethered aerostat (PS 11=) (same operational altitude, larger diameter to increase insolation and energy output to 50KWh. Link this solution to a system of wind for more energy generation and hydrogen for energy storage (the conversion of electricity generated from the aerostats through solar photovoltaic and a wind turbine? into hydrogen gas through electrolysis of water. This hydrogen can be stored in steel containers in smaller scales and can be used as fuel later in gas engines or hydrogen fuel cells.

Break up into what your solution includes (i.e., tethered aerostats, mobile aerostats,

database, etc.)

## Technical Overview

| PRODUCT | SPECIFICATION |
| --- | --- |
| Tethered Aerostat | Model – PS-11 Sphere diameter – Payload – 5 kg Power for Payload – 2 kVA / 1.6 kW Steel tether (total) - 2 Vectran tether – 1 Lifting gas – Operating altitude – 1.219 km Total suspended weight (fabric + payload + solar panels) Latitude – Operational Windspeed – 70km/h Total cost – |
| Mobile Aerostat | Model – Tsoro Sphere diameter - 20m Range – 120 km Payload – 5 kg Lifting gas – Airspeed – 24 km/h LED Screen (total) - 1 |
| Operating base |  |
| Electrolyzer | Model – HySTAT Alkaline Electrolyzer Cost – ($430 / kW) |

## Key Stakeholders

Primary Stakeholders: Entrepreneurs in the cities, the city’s governing authorities, and the genera Malawian populace.

Secondary Stakeholders/ Collaborators: Our solution is heavily dependent on:

(I) Funding:

(II) Local co-operation/ Participation: We will collaborate with three local universities, one in each of the three participating cities to help with popularization and easily accessible registration booths for the competition. These include:

- Lilongwe University of Agriculture and Natural Resources, Lilongwe

- University of Blantyre Synod, Blantyre

- University of Malawi, Zomba

## Expected Outcomes

After being selected as one of the three winners of the competition, teams/ ideas will be nurtured to their implementation stages through enrolment into a start-up incubator. They will be able to build networks with the other entrepreneurs, meet investors and collaborators, and nurse their ideas to implementation. Through implementation of the chosen ideas, we hope to achieve the bigger goal of combatting climate change and curbing its effects through the alleviation of poverty. Through long-term growth and progress of the competition and through successful implementation of vibrant entrepreneurial ideas, we hope to significantly reduce the unemployment numbers in urban and peri-urban Malawi.

## Cost Summary

| Item | Description | Quantity/Size | Cost ($) |
| --- | --- | --- | --- |
| Prize fund | Prize money for the three winners of the competition | 1 | 60,000 |
| Mobile aerostats | Cost of Production | 6 | 66,810 |
|  | Monthly Cost of Operation | 12 people | 3,024 |
| Panel of Judges | Compensation for Competition judges | 5 | 90,000 |

## Calculations - Cost of Production

|  | Quantity | Cost ($) |
| --- | --- | --- |
| Envelope | 15m diameter ~ 710m2 Estimated cost per square meter (HAB-1000) ~ $80 https://www.kaymont.com/product-page/hab-1000 | 56,800 |
| Solar panels | 20 Cost per panel - $500 | 10,000 |
| Light bulb | 1 | 10 |
|  |  | Total = 66,810 |

Calculations – Cost of Operation

Monthly cost per person=1.68 Minium daily wage×30=50.4 ×5=252

For 12 persons  two per mobile aerostat=246 ×12=  3,024

Calculations – Cost for Panel of Judges

Daily cost per judge=200×90 (3-month vetting period)=18,000

For entire panel  5 judges=18,000 ×5=  90,000

## Program Timeline

a) Product Acquisition/ Manufacturing Timeline: Our solution uses a simple customization of a previously developed mobile aerostat model named ‘Tsoro’ by Solar Ship Inc. Depending on time variations that may occur during the procurement of the design materials, we anticipate the development processes to take no longer than a year.

b) Competition timeline: Our pilot program will start with the simultaneous competition promotion and idea submission stage through the aforementioned means. This will be on for 6-months in each of the chosen cities or until 400 submissions are received - whichever comes first. This will be followed by a 3-month long review and idea selection process by the panel of judges.

## References

1. https://borgenproject.org/causes-of-poverty-in-malawi/

2. https://documents1.worldbank.org/curated/en/723781545072859945/pdf/malawi-scd-final-board-12-7-2018-12122018-636804216425880639.pdf

3. https://www.usaid.gov/malawi/fact-sheets/malawi-climate-change-fact-sheet#:~:text=However%2C%20due%20to%20high%20population,patterns%20are%20harming%20agricultural%20growth.

4. https://www.nationsencyclopedia.com/economies/Africa/Malawi-AGRICULTURE.html